Metal  Aid  Engineers

G-2, “Jains Brindavan”, 38, Brindavan Street, West Mambalam, Chennai-600033

Tel: 044-23711946, 24718292, 23701225 (Fax), Cell: 9444351612

Email: metalaidengg@rediffmail.com

MAGIC-WRAP-PIPE & JOINT REPAIR KIT

MAGIC-WRAP is resin impregnated water activated fiberglass cloth tape, easy to use, quick action answer, will repair broken parts, arrest pipeline /  Tank leakages etc. sets rock hard even under water.

Where to apply?

PVC, metals, rubber, fiberglass, plastics etc. Under-water, under - ground, and in corrosion protection area.

In Mediums

Water, acids, soda solutions, fuel oil with water, white spirit, toluene, xylene, diesel, gasoline, gas, air, steam, hydraulic fluids etc.

TYPICAL APPLICATION:      PIPE LEAK SEALING

Clean & Roughen the area to     Soak the MAGIC-WRAP in       Wrap MAGIC-WRAP the              Squeeze and mold MAGIC-WRAP

be repaired. Remove any oily    Water for 20 seconds                  entire roll, approximately 2”           in the direction of wrap, removing

or foreign particles                      (Using gloves)                            above and below the repair             excess water and spreading resin

area. Wrap as tightly as possible.   Until tacky to touch. (app.3 Mins)

A minimum of 8 layers of  ‘MAGIC-WRAP’ is required and shall be applied from right to left or from left to right in a spiraly maner around the pipe using 50% overlap. At the end of first pass, reverse the direction of the wrap and continue to apply until you have achieved specified number of layers. To achieve 8 layer application, apply 4 passes using a 50% overlap. Occasionaly twisting the MAGIC-WRAP as it is applied helps it conform tightly to the pipesurfac atelbows or at odd geometries. Allow  30 minutes for complete curing.

MULTIPLE ROLLS CAN BE USED FOR LARGER DIAMETER PIPES OR YOU CAN SPECIAL ORDER SIZES OF 4" X 20', 30', or 50'. AS A GENERAL RULEOF THUMB, USE 4-5 FT OF MAGIC-WRAP FOR EVERY INCH OF PIPE DIAMETER.

In applications wherever wrapping is not possible, like plugging a hole, MAGIC-WRAP should be unraveled and mold into a ball and plugged into the hole cand hand pressure can be given for about 5 minutes

Product Data

Tensile strength        Method: ASTM D3039

Hoop PSI                   56,700

Axial PSI                   21,600

Hardness                   Shore D

30 minutes                 47

2 hours                       76

24 hours                     83

Flexural                    Method: ASTM D790 – 3 point flex

Strength PSI               16,100

Shear Method:         ASTM D648

Strength PSI              7520

Coeff Of linear exp  Method: ASTM D648

CTLE                        1,06E-05

Dielectric strength     16000 volts

Heat resistance          1100°F

TYPICAL APPLICATIONS

- impact protection

- emergency repair

- cracked or corroded pipes

- irregular pipe fittings

- force mains and gravity pipe lines

- under water repair and splash zone

- joint and tee repair

PROPERTIES

- VOCs; None

- Pressure once cured: 20.6 BARS

- Working Time: 5 Minutes at 24ْCelsius

- Set time: 28 minutes at 21 ْCelsius

MAGIC WRAP-SELECTION AND PRICES

Pipe Diameter; ll Roll size recommended size; recom Unit Price per unit. ½” – 1”; 50 M.M X 1500 M.M; Box= 3 Tapes. 1 ¼” – 2”; 75 M.M X 2700 M.M.; Box=2 Tapes. > 2”; 100 M.M X 4500 M.M; Box= 1 Tape.

Chemical Resistance

| Immersion Chemical |  | Immersion Time |  | Effect | Immersion Chemical | Immersion Time | Effect |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Distilled Water |  |  | 60 Days | None | White Spirit | 60 Days | None |  |
| Sulphuric Acid, 10% |  |  | 60 Days | None | Toulene | 60 Days | None |  |
| Sulphuric Acid, 50% |  |  | 60 Days | None | Xylene | 60 Days | None |  |
| Hydrochloric Acid, 10% |  |  | 60 Days | None | Diesel Fuel | 60 Days | None |  |
| Soda Solution,Saturated |  |  | 60 Days | None | Gasoline | 60 Days | None |  |
| Sodium Hydroxide, 20% |  |  | 60 Days | None | Steam/Gas | 60 Days | None |  |
| Fuel Oil + % Water |  |  | 60 Days | None | Hydraulic Fluids | 60 Days | None |  |

TERMS OF SUPPLY

- Taxes: CST/Vat Extra as  applicable

- Packing & Forwarding: Extra @  3 %

- Freight 2% Extra.

- Insurance Extra to Your account

- Payment 100% Documents Through Bank/ Against Proforma Invoice

- Your Bank Charges to your Account and Our Bank Charges To Our Account

- Delivery: Immediate on receipt of clear purchase Order From Chennai By RPP

No L.D Clause Is Applicable.