Metal  Aid  Engineers

G-2, “Jains Brindavan”, 38, Brindavan Street, West Mambalam, Chennai-600033

Tel: 044-23711946, 24718292, 23701225 (Fax), Cell: 9444351612

Email: metalaidengg@rediffmail.com

MAGIC-WRAP-PIPE & JOINT REPAIR KIT

MAGIC-WRAP is resin impregnated water activated fiberglass cloth tape, easy to use, quick action answer, will repair broken parts, arrest pipeline /  Tank leakages etc. sets rock hard even under water.

Where to apply?

PVC, metals, rubber, fiberglass, plastics etc. Under-water, under - ground, and in corrosion protection area.

In Mediums

Water, acids, soda solutions, fuel oil with water, white spirit, toluene, xylene, diesel, gasoline, gas, air, steam, hydraulic fluids etc.

TYPICAL APPLICATION:      PIPE LEAK SEALING

Clean & Roughen the area to     Soak the MAGIC-WRAP in       Wrap MAGIC-WRAP the              Squeeze and mold MAGIC-WRAP

be repaired. Remove any oily    Water for 20 seconds                  entire roll, approximately 2”           in the direction of wrap, removing

or foreign particles                      (Using gloves)                            above and below the repair             excess water and spreading resin

area. Wrap as tightly as possible.   Until tacky to touch. (app.3 Mins)

A minimum of 8 layers of  ‘MAGIC-WRAP’ is required and shall be applied from right to left or from left to right in a spiraly maner around the pipe using 50% overlap. At the end of first pass, reverse the direction of the wrap and continue to apply until you have achieved specified number of layers. To achieve 8 layer application, apply 4 passes using a 50% overlap. Occasionaly twisting the MAGIC-WRAP as it is applied helps it conform tightly to the pipesurfac atelbows or at odd geometries. Allow  30 minutes for complete curing.

MULTIPLE ROLLS CAN BE USED FOR LARGER DIAMETER PIPES OR YOU CAN SPECIAL ORDER SIZES OF 4" X 20', 30', or 50'. AS A GENERAL RULEOF THUMB, USE 4-5 FT OF MAGIC-WRAP FOR EVERY INCH OF PIPE DIAMETER.

In applications wherever wrapping is not possible, like plugging a hole, MAGIC-WRAP should be unraveled and mold into a ball and plugged into the hole cand hand pressure can be given for about 5 minutes

Product Data

Tensile strength        Method: ASTM D3039

Hoop PSI                   56,700

Axial PSI                   21,600

Hardness                   Shore D

30 minutes                 47

2 hours                       76

24 hours                     83

Flexural                    Method: ASTM D790 – 3 point flex

Strength PSI               16,100

Shear Method:         ASTM D648              TYPICAL APPLICATIONS               PROPERTIES

Strength PSI              7520                            impact protection                                    VOCs; None

Coeff Of linear exp  Method: ASTM D648    emergency repair                                  Pressure once cured: 20.6 BARS

CTLE                        1,06E-05                      cracked or corroded pipes                     Working Time: 5 Mints at 24ْC

Dielectric strength     16000 volts                 irregular pipe fittings                            Set time: 30 Minutes Approx.

Heat resistance          1100°F                        force mains and gravity pipe lines

Under water repair and splash zone

Joint and tee repair

TYPICAL USAGE: INDUSTRY-WISE

COAL MINES: Coal preparation wash plants: MDISC, water, steam, slurry and sludge lines etc. Emergency repairs to leaking pipes.

GOLD MINES:  processing plant: chemical, sludge, slurry and by-product/waste pipes. Emergency repairs to chemical supply lines.

BAUXITE MINES:  processing plant: refinery/alumina section and smelter/aluminum section. Emergency repairs to sludge & chemical lines.

COAL, COPPER & ZINC SMELTING Processing: blast furnaces, water, steam, sludge and slurry lines. Emergency repairs.

STEEL SMELTER WORKS Processing: blast furnaces, water, steam and waste product pipes. Emergency repairs until shutdown.

STEEL SHEET & COIL MANUFACTURING Processing: steam and water pipes. Emergency repairs to burst pipes.

PETROLEUM Oil, gas and water supply lines. Emergency repairs to fuel, oil and gas lines during 24 hour shift.

SUGAR MILLS Corrosive fluid lines, abrasive product pipes, low pressure, water and steam lines. Emergency repairs to corrosive, water, steam and abrasive lines.

POWER STATIONS Ash transfer systems, pulverized fuel tubes and cooling water lines. Emergency repairs to transfer systems and lines.

MERCHANT SHIPPING Engine room. Emergency repairs while at sea to fuel, oil, and water and waste lines. Damage control within engine room and firefighting facilities.

Additional to leak repairs, Rapp-it can also be used as corrosion protection and noise reduction.

MAGIC WRAP-SELECTION AND PRICES

Pipe Diameter; ll Roll size recommended size; recom Unit Price per unit. ½” – 1”; 50 M.M X 1500 M.M; Box= 3 Tapes. 1 ¼” – 2”; 75 M.M X 2700 M.M.; Box=2 Tapes. > 2”; 100 M.M X 4500 M.M; Box= 1 Tape.

Chemical Resistance

| Immersion Chemical | Immersion Time | Effect | Immersion Chemical | Immersion Time | Effect |
| --- | --- | --- | --- | --- | --- |
| Distilled Water | 60 Days | None | White Spirit | 60 Days | None |
| Sulphuric Acid, 10% | 60 Days | None | Toulene | 60 Days | None |
| Sulphuric Acid, 50% | 60 Days | None | Xylene | 60 Days | None |
| Hydrochloric Acid, 10% | 60 Days | None | Diesel Fuel | 60 Days | None |
| Soda Solution,Saturated | 60 Days | None | Gasoline | 60 Days | None |
| Sodium Hydroxide, 20% | 60 Days | None | Steam/Gas | 60 Days | None |
| Fuel Oil + % Water | 60 Days | None | Hydraulic Fluids | 60 Days | None |